SOFTWARE DESCRIPTION

MULTI-TASKING MECHANISM

In order to handle asynchronous events in a timely manner, a

multi-tasking scheme is employed.  Each process which must occur

is defined as a TASK.  Each task has a task control block (TCB)

associated with it.  The TCB acts as a save area for the task's

status and registers during the time the task is not active.  The

TCB's are chained together by a forward chain pointer in each

TCB.  The TCBHDR word in storage points to the first TCB.

The function of a task is to look for requests and process them.

When there are no more requests for a task, that task gives up

control until more requests are generated for it.  The requests

are placed in a FIFO (first-in-first-out) queue (except for timer

requests, which are processed in the order the timers expire).  A

task may be responsible for one or more queues.

When the task must wait for more work or an external event, it

calls the WAIT routine.  The wait routine saves the task's

registers on the TCB stack and sets the task non-dispatchable.

At some later time an interrupt routine or another task will

issue a POST for the non-dispatchable TCB.  The post routine will

set the TCB dispatchable, and the task will be given control when

it becomes the highest priority ready task.

The dispatcher routine always starts with the TCB whose address

is in TCBHDR.  It checks each TCB in the TCB chain in turn,

looking for a TCB that is dispatchable.  When a dispatchable TCB

is found, its registers are reloaded from its TCB stack, and

control is passed to the task at the instruction following the

last interruption.

If a POST is issued for a task that is not waiting, the POST flag

is set for the TCB anyway.  If the task later issues a WAIT, the

wait routine will reset the POST flag, but leave the task

dispatchable.  This prevents a POST from getting lost.  All

interrupt routines use the INTSAV and INTRET routines to save

registers and return from the interrupt.  INTSAV resets the

system-wide post flag (SYSPST).  The post routine sets SYSPST if

any TCB is posted.  INTRET checks the SYSPST flag.  If SYSPST has

not been set, INTRET returns to the interrupted task.  If SYSPST

has been set, INTRET passes control to the dispatcher.  If the

post was for a higher priority task, the posted task will get

dispatched instead of the interrupted task.

A task's priority is based solely on its position in the TCB

chain.  The first task is the highest priority; the last task is

the lowest priority.  There is currently no mechanism to change

the order of the TCB chain.

The last task on the TCB chain is the dummy wait task.  It

consists of a HALT instruction and a jump to the HALT.  This task

is always dispatchable, since it never issues a wait.  If all the

other tasks are non-dispatchable, the dispatcher will select the

dummy wait task.  When the HALT instruction is executed, the HALT

status pin on the CPU chip is active.  An LED or meter connected

to this pin gives a visual indication of how busy the CPU is.

Since a task can be interrupted at any point, all code must be

re-entrant or run disabled for interrupts.  Serialization of

queues is maintained by disabling interrupts during queue

manipulation.  Enablement and disablement are controlled by the

system disablement count (SYSDIS).  Each disable increments the

count.  Each enable decrements the count.  When the count reaches

zero, the ENABLE routine physically enables the CPU for

interrupts.

Queues are designed to provide a multi-threaded means of handling

requests.  This means that one request does not have to wait for

other unrelated requests to be processed.  Typically, a task is

responsible for a logical function.  The task controls several

hardware devices which provide the function.  The logical

requests are placed on the task's input queue by the requestor.

The task removes the requests from its input queue and places

them on one of its device queues.  When the device becomes

available, the request is removed from the device queue and the

device is activated.

Queues are manipulated by changing the chain pointers in the

elements and headers.  Data areas are never moved or copied.  A

queue header holds the address of the first and last entries in

the queue.  Each entry contains a forward chain pointer to the

next entry.  The last entry contains a zero chain pointer.

#####################################################################################