FILENAME:  BCLOCK.DOC  by Don McKenzie.                Updated Oct 28, 1993

Build a BINARY LED CLOCK with only a PBUFF_ZLOAD Kit, 17 LEDs, 17 Resistors,

3 Switches (or IDC type male posts), and a box to put it all into.

�����������������������������������������������������������������������ͻ

� (12/24)�ķ                                                            �

�         16  8  4  2  1    32 16  8  4  2  1     32 16  8  4  2  1     �

�  17    ����������������������������������������������������������Ŀ   �

� LEDS   � O  O  O  O  O  �  O  O  O  O  O  O  �   O  O  O  O  O  O �   �

�        ������������������������������������������������������������   �

�             HOURS              MINUTES               SECONDS          �

�               �����Ŀ                                                 �

�               �������       �����������������������������������ķ     �

�      SWITCHES �������       � BINARY LED CLOCK By Don McKenzie. �     �

�               �������       �����������������������������������Ľ     �

�                S - +                                 Oct 93(c)        �

�����������������������������������������������������������������������ͼ

In the October 93 edition of Silicon Chip, I spotted a binary clock project

that used DIP switches to set the time, and GALs for control and display

logic. It costs $75 for the full kit minus plugpack, and $25 for an optional

folded aluminum stand.

Well, I wonder what it's like to have a binary clock? Hang on! Couldn't I do

this easily with a ZLOAD kit and a handful of components? And if I don't

like it, I can make something else out of the kit.

And I know I can do it for under $75. A ZLOAD blank board only costs $35. I

don't need an EPROM burner for ZLOAD to test and run the circuit, but it

will cost me only an EPROM if I have a burner and wish to box the project

and hang it up on the wall, or put it on my desk to amuse my computer type

friends.

The circuit is easy. A ZLOAD with 6264, no Dynamic Ram, no 74LS00, no 555

timer, the pause pads shorted, a 26 pin male IDC type header soldered into

position J2, and a 26 wire flat ribbon cable crimped to a 26 pin female IDC

header.

On the other end of the cable you will need the items described in the first

sentence of this article. To mount these, I used a TANDY Cat 276-147 $6.95

component PC board cut in half length ways with a pair of tin snips. You may

wish to install it all into a pretty box or mount it into an acrylic sheet

housing.

A 3.579545 Mhz crystal was used for this project. Any value IE: 4.9152,

6.144, (Z80B) could be used by adjusting the software timing values. This

could be a small project in itself. I will probably find these values

myself in due course.

When you choose what colour LED you wish to use for your binary clock it is

most important that the LEDs are all the same type number so that there are

no inconsistencies of colour or brightness between each LED. It is also very

important that the LEDs have an intensity somewhere between 7 - 10 mCa

(milli Candela) and a viewing angle of 36 degrees or greater.

Most RED LEDs that you would buy off the shelf would be 0.8 mCa which is

totally unsuitable for the job whereas most GREEN or ORANGE are 8.7 mCa (the

range is very broad starting at about 0.8 and going to 350 and even higher

mCa), do not buy the high intensity type either as they have a typical

viewing angle of only 15 degrees which also is unsuitable. If your store

owner cannot tell you the intensity and viewing angle of the LEDs then don't

buy them.

Typical examples of suitable LEDs are the LITON brand LTL-4223 (RED),

LTL-4233 (GREEN) and LTL-4293 (ORANGE) which all have an intensity of 8.7

mCa and a viewing angle of 36 degrees. One last warning, buy ALL your LEDs

at once, as you may not be able to get the same type again at a later date.

The data section of Dick Smith Electronics current Catalogue has a very good

run down on Intensity values for the LEDs they stock. You will find suitable

types in small quantities at reasonable prices.

I soldered my 17 LEDs, 17 resistors, 26 pin male header, and 6 pin male

header for the switches into the board and wired it all up. The 26 pin male

header mates up with the ZLOAD J2 header via a length of flat ribbon cable

and crimp connectors. The anode end of the LEDs connect to +5Volts. The

cathode ends connect to each resistor.

I used an IDC type male block of ten pins for the switches, and removed the

alternate rows leaving 6 pins so that I had .1" between the pins and .2"

between each switch. I was able to program the switches using a small

screwdriver. As I did all of my software development and used only the

screwdriver, I'm sure you don't need real switches for this project. Switch

debouncing is all done with software. Even a Biro will do to adjust the

clock.

CIRCUIT:

�����������������������������������������������������������������������ͻ

�             HOURS              MINUTES               SECONDS          �

�  +5V ����������������������������������������������������������Ŀ     �

� Anode +����������������������������������������������������������Ŀ+  �

�17 LEDs � O  O  O  O  O  �  O  O  O  O  O  O  �   O  O  O  O  O  O �   �

�Cathode-������������������������������������������������������������-  �

�         ����������   ������������    ������������    �                                     ��

�      R17� �� �� �� �� �   � �� �� �� �� �� �    � �� �� �� �� �� �R1  �                                     � �R1

�    270  ���������������   ������������������    ������������������    �                                     ���

�    OHMS  �  �  �  �  �     �  �  �  �  �  �      �  �  �  �  �  �     �

�          �  �  �  �  �     �  �  �  �  �  �      �  �  �  �  �  �     �

�          � PA7 � PA5 �    PA3 � PA1 � PB7 �     PB5 � PB3 � PB1 �     �

�         PC2   PA6   PA4      PA2   PA0   PB6       PB4   PB2   PB0    �

�                                                                       �

�               SET  -   +                                              �

�     GND ��������������Ŀ                                              �

�              �����������Ŀ                         �������������Ŀ    �

�     3        � � � � � � �                         �OOOOOOOOOOOO��1   �

�  SWITCHES    � � � � � � �                      26 �OOOOOOOOOOOOO�    �

�              �������������                         ���������������    �

�                �   �   �                         26 PIN MALE HEADER   �

�                �   �   �                                              �

�               PC4 PC5 PC6                                             �

�����������������������������������������������������������������������ͼ

�����������������������������������������������������������������������Ŀ

� ZLOAD J2 PINOUT:                    J2 & J7 are the same.             �

�                        26                                             �

�                        ���Ŀ                                          �

�                     +5V�O O�+5V                                       �

�                     GND�O O�GND                                       �

�        MINUS SWITCH PC5�O O�PC6 PLUS SWITCH (MAKE SURE PAUSE SHORTED) �

�            NOT USED ---�O O�PC2������MSB HOURS                        �

�                   ��PA7�O O�PB7Ŀ                                     �

�  LOWER 4 BITS ������PA6�O O�PB6������LOWER 2 BITS MINUTES             �

�  HOURS            ��PA5�O O�PB5Ŀ                                     �

�                   ��PA4�O O�PB4Ĵ                                     �

�                   ��PA3�O O�PB3������SECONDS                          �

�  UPPER 4 BITS ������PA2�O O�PB2Ĵ                                     �

�  MINUTES          ��PA1�O O�PB1Ĵ                                     �

�                   ��PA0�O O�PB0��                                     �

�          SET SWITCH PC4�O ��STB (1 SECOND PULSE AVAILABLE)            �

�                        �����                                          �

�                           1                                           �

�������������������������������������������������������������������������

Once you have the circuit wired up, the file BCLOCK.HEX can be down line

loaded via your PC parallel printer port to ZLOAD. Read ZLOAD.ASS for full

details on this.

�����������������������������Ŀ

�GO.BAT �COPY BCLOCK.HEX LPT1 �

�       �SETPRT               �

�������������������������������

Above is a simple batch file to load the Binary clock program into ZLOAD.

The SETPRT program (included on disk) will set the LPT1: data lines high

after the load so that the clock can be used while still connected to your

PCs printer port.

You should now be able to test and run the clock. Read on...

Binary Clock Operation and Time settings:

On Power-up, the left hand side LED flashes at 2 CPS until the Set Switch is

pressed. This is done to show that the program is now running, but is

running from a power fail condition.

This is the same as the flashing "00:00:00" on todays Electronic gadgets

such as VCRs, Microwave Ovens etc. I used only one LED to save power.

If the Set Switch is pressed after power up, the clock starts counting from

time "00:00:00".

There are 3 switches, and from left to right are:

(1) Set Switch.

(2) Minus Switch. (Also Hold)

(3) Plus Switch.  (Also Adjust Speed.)

Setting Time:

HOURS>> When the clock is running, press Set. The Hour LEDs will flash 4

times, then the current hours setting will be displayed. The Plus and Minus

switches can now be used to adjust the hours up or down. The adjusted

display changes at a rate of 250ms.

MINUTES>> If you don't need to change the hours, Press Set again. The Minute

LEDs will now flash 4 times, and can be set in the same manner as the Hours.

SECONDS>> Press Set once more. Seconds can be set the same as Hours and

Minutes.

Exiting Time Setting>> Press Set again. Seconds LEDs flash 2 times (1 second

delay), then clock continues with your new settings.

As this is a binary clock, I found the easiest way of doing my mental

arithmetic was to set hours and minutes up in advance and use a Hold feature

to start the clock at Zero seconds.

HOLD>> When the clock is running, press Hold (Minus Switch). The Seconds

LEDs flash four times, and the Seconds are set to Zero. This stops the clock

at its current Hours and Minutes settings. When Hold is released, the clock

counts from Zero Seconds.

12 HOUR>> With the COPY switch or pads open, the clock is in the 12 Hour

mode. It counts to 11:59:59 as normal, then sets the MSB Hour LED on when 12

is reached. This is the PM LED. Therefore 12:00:00 shows up as 00:00:00 with

the PM LED on. This really helps when you are trying to read a binary clock.

24 HOUR>> With the COPY switch or pads closed, the clock is in the 24 Hour

mode. It counts to 23:59:59 as normal, and at 24:00:00 resets to 00:00:00.

ADJUST SPEED>> When the clock is running, press Adjust Speed (Plus Switch).

The Minutes LEDs flash four times, and a 16 bit Binary value is displayed in

the right hand side 16 LEDs. This is the Fine tune delay value which is

currently set at 8660Hex for a 3.579545Mhz Crystal. This value may vary

slightly as I am still playing around with the software.

The Plus and Minus Switches can be used to adjust the speed of the clock. A

larger value will slow the clock down. A smaller value will speed up the

clock. Press Set to exit this mode and run the clock.

This project uses a crystal for timing, however I have included the speed

adjust feature to overcome inaccurate crystals and possible extreme

temperature variations. When the correct Hex value is found, it should be

recorded for the next power loss situation.

Alternatively, the BCLOCK.Z8T Source code can be re-assembled with your new

"VALUE2" changes, or the EPROM can be patched at address 30H for LSB, and

31H for MSB of your new 16 bit fine tune timing value.

Initially, four equal values are set up in the software as four 250ms timing

loops. The last loop has the "VALUE2" included. This is done only in one

loop so that a minimum number of executable instructions can be used to trim

the timing.

DATA LED>> If fitted, is ignored. The 8255 pin that supports the data LED is

now re-programmed as an input signal. The data LED will be on until you

Press the Minus Switch.

8255 PROGRAMMING>> The value loaded to the status register is 88H. This sets

up the 8255 as follows:

PORTA 8 BITS OUTPUT             PA0-PA7

PORTB 8 BITS OUTPUT             PB0-PB7

PORTC LOWER 4 BITS OUTPUT       PC0-PC3

PORTC UPPER 4 BITS INPUT        PC4-PC7

I am sending out a narrow 1 second low going pulse on pin 1 of J2 of ZLOAD.

This could be very handy for some people. It wouldn't be hard to do the same

thing with Minutes and Hours pulsing out of the unused output pins of the

8255 namely, PC0, PC1, & PC3. It could be a very useful micro-controlled

timebase for that special job.

And where do you get it? The software including Z80 source code is now free

with every PBUFF_ZLOAD kit. This file is the full operating and assembly

instructions for the Binary Clock.

That's it. Thinking about doing a Duo-Decimal clock now.

Any constructive comments or criticism would be appreciated. Don.